- Building

  - Base structure reinforced concrete; 99’ outside diameter with 13” thick walls.

  - Base structure 35’ high from grade 0.0’

  - Height of dome from top of concrete to top of shutter 60’

  - Total height of building from grade 0.0’. is 95’

- Dome

  - Outside diameter of dome 96’ 6”

  - Outside dome skin ¼” steel plate spherically shaped plates.

  - Thickness of dome shell 3’ with 2’ 9” air space for insulation.

  - Inner finish of dome is corrugated aluminum shaped sheet with 3” of aluminum foil for insulation.

  - Dome is supported on 30 trucks, each with two wheels.

  - Main arch girders are 5’ deep with 18 inch channels for top and bottom flanges and ¾” thick web plate.

  - Weight of dome approximately 200 tons.

- Telescope

  - Diameter of the primary mirror is 120” = 10’

  - Focal length 600” = 50’

  - Camera speed = f/5

  - Auxiliary equipment:

    - Prime focus camera for 5” x 7” plates.

    - Prime focus spectrograph with 4” square gratings.

    - Prime focus photometer.

  - Cassegrain spectrograph f/15

  - Coude Spectrograph:

    - 3-mirror system f/36

    - 5-mirror system f/38.7

  - Mirrors

    - 1st mirror 120” dia. X 16” thick (concave) 7740 lbs. weight.

    - 2nd mirror 31” dia. x 4” thick (convex) 240 lbs. weight.

    - 3rd mirror 24” wide x 50” long x 9” thick (flat) 1000 lbs. weight.

    - 4th mirror 31” dia. X 5 ½” thick (flat) 326 lbs. weight.

    - 5th mirror 44” dia. X 6” thick (flat) 560 lbs. weight.

- Telescope Tube

  - Length 51’ 6” including old prime focus cage.

  - Old Prime focus cage is 12” O.D. x 8’ high, 5000 lbs. weight

  - Total weight 45 tons = 90,000 lbs. includes 12,000 lb. mirror cell and 7740 lb. mirror.

  - Tubes upper truss 8” O.D. x 6 5/8 I.D. seamless tubes – Lower truss 8” O.D. x 7 ½” I.D. seamless tubes.

  - Balance weights – 5 tons – 10,000 lbs. (in 4 units).

- Fork

  - Two fork arms – 12 tons each (24,000 lbs.).

  - One yoke – 33 tons (66,000 lbs.).

  - Total weight – 57 tons (114,000 lbs.).

    - Plate thickness varies from 1 ½” at center of yoke to ⅝” at arm tips.

    - Bolted joints between fork arms and yoke, and yoke and polar axle are attached with pre-stressed bolts at approximately 70,000 psi.

- Polar Axle (all cast steel)

  - North journal section