Lick Observatory

Mt. Hamilton, California

Basic Dimensions and Information Regarding 120-inch Telescope

- BUILDING

  - Base structure reinforced concrete; 99 ft. outside diameter with 18 in. walls

  - Base structure 35 ft. high from grade 0.0 ft.

  - Height of dome from top of concrete to top of shutter 60 ft.

  - Total height of building from grade 0.0 ft. = 95 ft.

- DOME

  - Outside diameter of dome 96 ft. 6 in.

  - Outside dome cover ¼-inch steel spherically shaped plates

  - thickness of dome shell 3 ft. with 2 ft. 9 in. air space for insulation

  - Inner finish of dome corrugated aluminum shaped sheet with 3 inches of aluminum foil for insulation.

  - Dome supported on thirty (30) 2-wheeled trucks.

  - Main arch girders 5 ft. deep with 18 in. channels for top and bottom flanges and ¾-in. web plate.

  - Weight of dome approximately 200 tons.

- TELESCOPE

  - Diameter of main mirror 120 in.(10 ft.)

  - Focal length 600 in. (50 ft.)

  - Camera speed f/5

  - Auxiliary equipment:

    - prime focus camera for 5” and 7” plates

    - prime focus spectrograph with 4 in. square gratings

    - prime focus photometer

  - Cassegrain spectrograph f/15

  - Coude spectrograph:

(1) 3-mirror system f/36

(2) 5-mirror system f/38.7

  - Mirrors:

    - 1st mirror 120” diameter x 16” thick (concave), 7740 lbs. weight

    - 2nd mirror 31”diameter x 4” thick (convex), 240 lbs. weight

    - 3rd mirror 24” wide x 50” long x 9” thick (flat), 1000 lbs. weight

    - 4th mirror 31” diameter x 5-1/2” thick (flat), 326 lbs. weight

    - 5th mirror 44” diameter x 6” thick (flat), 560 lbs. weight

- TELESCOPE TUBE

  - Length 51 ft. 6 in., including prime focus cage

  - Prime focus cage 12 ft. OD x 8 ft. high, 5000 lbs. weight

  - Total weight 45 tons = 90,000 lbs., includes 12,000 lb. mirror cell and 7740 lb. mirror.

  - Tubes: upper truss 8” OD x 6-5/8” ID, seamless tubes; lower truss 8” OD x 7-1/2”  ID, seamless tubes.

  - Balance weights 5 tons = 10,000 lbs. (in 4 units)

- FORK

  - Two fork arms - 12 tons each (24,000 lbs)

  - One yoke – 33 tons (66,000 lbs).

  - Total weight – 57 tons (114,000 lbs)

(1)  Plate thickness varies from 1-1/2” at center of yoke to 5/8” at arm tips.

(2)  Bolted joints between fork arms and yoke, and yoke and polar axle are 	with pre-stressed bolts at approximately 70,000 psi.

- POLAR AXLE (all cast steel)

  - North journal section – 84” OD, 4” wall thickness x 33” long

  - North center section – 82-3/4” OD, 2-1/2” wall thickness x 89” long

  - South center section – 82” OD, 1-3/4” wall thickness x 95” long

  - South journal section – 48” OD, 2-1/2” wall thickness x 62” long

This section includes south journal and carries thrust pad flange and right 	ascension gear journal.

  - Total weight – 34 tons (68,000 lbs.)

  - Total length – 23 ft. 3 in. (279 in.)

- OIL PAD BEARINGS

  - Oil film thickness - .002 to .003 in.

  - Oil pressure – approximately 800-850 psi

  - Pad diameters:

    - North (2 pads) – 25-3/4” OD – 24” diameter

    - South (2 pads) – 16-3/4” OD – 15” effective diameter

    - Thrust (1pad) – 24” OD – 22-1/4” effective diameter

  - OD includes oil recovery channel

  - Oil flow per pad – approximately 1 gallon/minute

  - Oil viscosity approximately SAE 10 with high viscosity index.

- RIGHT ASCENSION WORM GEAR

  - Consists of two steel gears on single hub.

    - One gear for fast setting of telescope (slew speed)

    - One gear for celestial drive of telescope (tracking speed)

  - Each gear has 720 teeth; 143” OD; .625 circular pitch

  - Worms 5-1/2” OD, single thread, nickel bronze

- DECLINATION SPUR GEAR

  - A weldment with 605 teeth

  - 121” pitch diameter and a 5 diametral pitch, 4” face width.

  - Pinion 60 teeth with a 12” pitch diameter.

- DRIVES

  - Right ascension

    - slewing – 1-1/2 HP – speed 45 deg. per minute

    - set rate – ¼ HP – 77 seconds of arc per second of time (adjustable)

    - guide rate – 1/8 HP – 2-1/4 sec. of arc per second of time (adjustable)

    - tracking rate – 1/25 HP – 15 sec. of arc per second of time (adjustable)

  - Declination

    - slewing – ¾ HP – speed 45 deg. per minute

    - set rate – ¼ HP – 77 sec of arc per second of time (adjustable)

    - guide rate – 1/8 HP – 2-1/4 sec of arc per second of time (adjustable)

    - lunar rate – 1/8 HP - .33 sec of arc per second of time (adjustable)

  - All rates except tracking and slewing drive through Graham variable speed reducers giving remotely controlled rate adjustments.

- COUDE SPECTROGRAPH

  - Collimating mirrors 12” diameter x 2” thick, and 15” diameter x 3” thick (future).

  - Collimated beam 6” diameter in one position and 9” diameter in extended position.

  - Gratings – approximately 8” x 10” – 15,000 and 22,500 lines/inch.

  - Dispersion range 32.8 A/mm to .85 A/mm.

  - Cameras:

    - 20” focal length mirror dia. 29-1/2” x 7” – Plate holder size 1”x 6”

    - 40” focal length mirror dia. 31-1/2” x 5” – Plate holder size 1-1/4” x 8”

    - 80” focal length mirror dia. 36” x 8” – Plate holder size 2” x 12”

    - 160” focal length mirror dia. 50” x 91” – Plate holder size 2” x 24”