| Maintenance Performed | Hz | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dome Drive |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome tack – lube | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome tuck, zerks, FF holes - lube | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome drive assembly – lube / fluid check | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check drive fluid couplings | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome Shutter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Upper shutter gearbox – lube / fluid check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Upper shutter tracks & wheels - grease & zerks | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect shutter cables for wear & tension | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower shutter wheels – zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower shutter drive & tracks – lube/fluid check | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean dome encoder tape | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean dome encoder optics | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube dome encoder pantograph joints | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect slip-rings – service as needed | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect & service dome outhaul winch | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Service bridge crane – lube/fluid check/zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| PF elevator drive gearbox – lube / fluid check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| PF elevator track – grease | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect PF elevator cables for wear & tension | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube fork arch rails and screws | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube 5th mirror ball screws | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube 5th mirror chain | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check 5th mirror gear box | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Mirror handling fixture – fluid check/zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |

| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Performed | Hz | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| Telescope |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Declination gearbox – lube / fluid check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec. tangent arm gearbox – fluid check | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec. counterweight – zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Mirror cover drive box & chain | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Mirror cover zerks and gears | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dust off mirror cover and secondary | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Co2 clean primary | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Coude′ Camera Room |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coude′ Slit Room |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating Facilities |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kinney vacuum pump oil level check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Kinney vacuum pump belt check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Kinney vacuum pump seals & motor zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Roots blower oil level check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Roots blower belts check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Diffusion pumps fluid level check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Diffusion booster pumps fluid level check | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean & grease tank access seal | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean tank side viewing ports | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dust off tank | 2 |  |  |  |  |  |  |  |  |  |  |  |  |

Notes:

| Maintenance Performed | Hz | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Air Compressors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check belt condition & tension | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Drain condensation from compressor tanks | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Change compressor oil | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Conditioning |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean/change CAT control room ac filter | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean/change AO control room ac filter | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Change Shane control room ac filters | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Coude′ camera room air filters | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Polar room electrostatic filters & check belt | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Shop |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean and lube shop mill | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean and lube shop lathe | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Polar Axle Room |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Replace oil rags / polar axle room | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check cradle shift makeup reservoir level | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Oil Pumps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clean around lube pumps/tanks | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Pad oil filter change – spin on cartridge (two) | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Service oil pumps & motors – zerks | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check pad & worm oil tank level | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Compressors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oil change | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Compressor belts check | 6 |  |  |  |  |  |  |  |  |  |  |  |  |
| Drain condensation from compressor tank | 12 |  |  |  |  |  |  |  |  |  |  |  |  |

Notes:

| Maintenance Performed | Hz | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Manlift |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check battery fluid | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check handrail setscrews (safety) | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check brake fluid | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| General safety and operation check of manlift | 6 |  |  |  |  |  |  |  |  |  |  |  |  |

Notes: