| Maintenance Performed | Hz | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dome Elevator Floor |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pump zerks & seal packing | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease tower weight guides | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease tower sheave bearing zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check tower cables for wear | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease floor rack & pinion gears | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease floor pinion shaft bearings | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check water tank level | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check ram packing for leaks | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome Shutter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Top shutter pivot zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Shutter wheel zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease shutter track | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check shutter drive belts | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease shutter drive | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dome Drive & Wheels |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease track | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease wheel bearing zerks | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check wheel bearing shaft centering | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check wheel bearings for casting cracks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease hurricane wheel zerks | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check drive fluid couplings (keep fluid low) | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease drive zerks | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check drive gearbox fluid level | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspect slip-rings – service as needed | 1 |  |  |  |  |  |  |  |  |  |  |  |  |

| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance Performed | Hz | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| Telescope |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grease drive tracking gear and drive | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube RA and DEC axles (brass oil hole plugs) | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Service RA clamp – hydraulics and zerks | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check RA clamp ability to hold and release | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube telescope hand wheels | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube tailpiece rotation joint (small notches) | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dust off telescope and mount | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Co2 clean front & back lens (open cell vent) | 6 |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Compressor |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oil change | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Compressor belts check | 6 |  |  |  |  |  |  |  |  |  |  |  |  |
| Drain condensation from compressor tank | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Windscreen |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube track guides & drive zerks | 1 |  |  |  |  |  |  |  |  |  |  |  |  |

Notes: